LANE DETECTION

IMPLEMENTATION GUIDE

Real-time perception pipeline for autonomous ground vehicles

ROS 2 Humble  •  Luxonis OAK-D Stereo Camera

Version 1.0

Date: July 2026

Table of Contents

1.  Introduction

2.  System Overview

3.  Prerequisites

4.  Building the DepthAI ROS Driver

5.  Building the Lane Detection Package

6.  Running the Lane Detection Pipeline

7.  Lane Detection Algorithm Implementation

7.1  Image Acquisition

7.2  Camera Undistortion

7.3  Perspective Transformation

7.4  Yellow Lane Detection

7.5  Dashed Lane Detection

7.6  Polynomial Curve Fitting

7.7  Lane Tracking

7.8  Lane Centre Estimation

7.9  Pixel Error Computation

7.10  Path Generation

7.11  ROS Topics Published

7.12  Debug Visualization

8.  Expected Data Flow

9.  Execution Summary

10.  Expected Results

11.  Conclusion

# 1.  Introduction

This document presents the implementation and execution procedure of a real-time lane detection system developed on ROS 2 Humble using the Luxonis OAK-D stereo camera.

The system employs the DepthAI ROS driver to acquire live RGB images, which are processed by the lane_detection node to identify lane markings, estimate the lane centre, compute vehicle lateral error, and publish navigation information. The processed results are visualized in real time using rqt Image View, enabling validation of the complete perception pipeline.

The implementation is designed for autonomous navigation applications where robust lane estimation is required for downstream motion planning and vehicle control.

# 2.  System Overview

The complete software architecture is illustrated below.

Figure 1 — End-to-end software architecture

## System Components

| Component | Description |
| --- | --- |
| OAK-D Camera | Captures live RGB images |
| DepthAI ROS Driver | Publishes camera topics |
| lane_detection | Detects lane markings and generates the driving path |
| rqt Image View | Displays the processed debug output |

# 3.  Prerequisites

Ensure the following software is installed before building the workspace.

| Software | Version |
| --- | --- |
| Ubuntu | 22.04 |
| ROS | Humble |
| OpenCV | 4.x |
| Python | 3.10 |
| Colcon | Latest |
| DepthAI ROS Driver | Installed |
| OAK-D Camera | Connected |

# 4.  Building the DepthAI ROS Driver

Clone the repository.

cd ~/ros2_ws/src git clone https://github.com/luxonis/depthai-ros.git

Return to the workspace and build.

cd ~/ros2_ws colcon build --symlink-install

Source the workspace.

source install/setup.bash

# 5.  Building the Lane Detection Package

Copy the package into ros2_ws/src.

Build only the lane_detection package:

cd ~/ros2_ws colcon build --packages-select lane_detection --symlink-install

...or build the entire workspace:

colcon build --symlink-install

Source the workspace.

source install/setup.bash

# 6.  Running the Lane Detection Pipeline

The implementation requires three terminals running concurrently.

## Terminal 1 — Launch the camera

source ~/ros2_ws/install/setup.bash ros2 launch depthai_ros_driver_v3 driver.launch.py

## Terminal 2 — Run the lane detection node

source ~/ros2_ws/install/setup.bash ros2 run lane_detection lane_detection

## Terminal 3 — Launch the visualizer

rqt

Navigate to:

- Plugins → Visualization → Image View

Select /lane/debug_image. The live processed lane visualization will be displayed.

# 7.  Lane Detection Algorithm Implementation

The lane detection algorithm consists of several computer vision stages executed sequentially for every incoming camera frame.

## 7.1  Image Acquisition

The node subscribes to /oak/rgb/image_raw using sensor_msgs/Image. The image is converted into an OpenCV matrix using CvBridge.

## 7.2  Camera Undistortion

The OAK-D camera calibration parameters are used to remove radial lens distortion. The OpenCV function cv2.undistort() generates a geometrically corrected image.

## 7.3  Perspective Transformation

The corrected image is converted into a bird's-eye view. The transformation uses source points and destination points through:

cv2.getPerspectiveTransform() cv2.warpPerspective()

This simplifies lane estimation by making lane boundaries approximately parallel.

## 7.4  Yellow Lane Detection

The left lane boundary is extracted using HSV colour segmentation. Processing stages include:

- HSV conversion

- Colour thresholding

- Morphological opening

- Morphological closing

- Contour filtering

Only contours larger than a specified minimum area are retained.

## 7.5  Dashed Lane Detection

The centre dashed lane is detected independently. The processing stages are:

- CLAHE enhancement

- Adaptive thresholding

- Gaussian blur

- Canny edge detection

- Morphological filtering

- Contour validation

Contour validation considers area, elongation, and solidity.

## 7.6  Polynomial Curve Fitting

Detected lane pixels are approximated using a second-order polynomial, x = Ay² + By + C, fitted with numpy.polyfit().

Separate polynomials are maintained for the yellow lane, the centre lane, and the left lane centre. Historical coefficients are averaged to smooth the detected lane.

## 7.7  Lane Tracking

The algorithm maintains historical lane estimates. If a lane cannot be detected:

- Previous polynomial coefficients are reused

- Sudden lane jumps are rejected

- Recovery mode searches the full image until valid lanes reappear

This improves robustness under noisy conditions.

## 7.8  Lane Centre Estimation

The driving path is computed as the midpoint between the yellow lane and the centre dashed lane. This produces the Left Lane Centre (LLC), which represents the desired driving trajectory.

## 7.9  Pixel Error Computation

Vehicle lateral deviation is calculated as:

Pixel Error = Camera Centre - Lane Centre

| Error | Vehicle Position |
| --- | --- |
| Positive | Vehicle is left of lane centre |
| Negative | Vehicle is right of lane centre |
| Zero | Vehicle centred |

## 7.10  Path Generation

The lane centre is converted into the ROS base_link coordinate frame. The node publishes nav_msgs/Path, which can be directly consumed by:

- Pure Pursuit

- MPC

- Stanley Controller

- Other path-following algorithms

## 7.11  ROS Topics Published

| Topic | Type | Description |
| --- | --- | --- |
| /lane/debug_image | sensor_msgs/Image | Debug visualization |
| /lane/pixel_error | Float32 | Vehicle lateral error |
| /lane/polynomial_coeffs | Float64MultiArray | Lane polynomial coefficients |
| /lane/status | String | Lane detection status |
| /lane/left_lane_centre_path | nav_msgs/Path | Generated driving path |

## 7.12  Debug Visualization

The node generates a comprehensive six-panel debug visualization showing each stage of the pipeline:

- Undistorted RGB image

- Bird's-eye view

- Yellow lane segmentation

- Dashed lane segmentation

- Polynomial curve fitting

- Final lane overlay

This visualization enables developers to inspect each processing stage independently.

# 8.  Expected Data Flow

The diagram below summarizes the complete per-frame processing pipeline, from raw image acquisition to published navigation data.

Figure 2 — Per-frame data flow through the perception pipeline

# 9.  Execution Summary

| Step | Command |
| --- | --- |
| Build workspace | colcon build --symlink-install |
| Source workspace | source install/setup.bash |
| Launch camera | ros2 launch depthai_ros_driver_v3 driver.launch.py |
| Run node | ros2 run lane_detection lane_detection |
| View output | rqt -> /lane/debug_image |

# 10.  Expected Results

After successful execution:

- The OAK-D camera continuously streams RGB images.

- The lane_detection node performs real-time lane detection.

- Yellow lane boundaries and centre lane markings are extracted.

- Second-order polynomial curves are fitted to the detected lanes.

- The lane centre path is estimated.

- Vehicle lateral deviation is calculated and published.

- Debug visualization is available through /lane/debug_image.

- Navigation information is published for downstream autonomous driving modules.

# 11.  Conclusion

The implemented lane detection system provides a complete real-time perception pipeline for autonomous ground vehicles operating under ROS 2 Humble. By integrating camera calibration, perspective transformation, colour-based lane extraction, polynomial curve fitting, temporal filtering, and path generation, the system produces reliable lane geometry suitable for vehicle control and path planning.

The modular ROS 2 architecture and rich debugging outputs make the implementation straightforward to validate, extend, and integrate with higher-level navigation algorithms such as Pure Pursuit, Stanley, or Model Predictive Control (MPC).