Traffic Sign DETECTION

PACKAGE DOCUMENTATION

ROS 2 object detection using YOLOv8 and an OAK-D stereo camera

Developer Documentation

Version 1.0

Date: July 2026

Table of Contents

1.  Introduction

2.  System Architecture

3.  Installation

4.  Running the Package

5.  Package Structure

6.  Package Responsibilities

7.  ROS Topics

8.  Custom Messages

9.  Detection Pipeline

10.  Distance Estimation

11.  Future Improvements

# 1.  Introduction

## Purpose

This package performs real-time traffic sign detection using a custom-trained YOLOv8 model running on an NVIDIA Jetson platform. It subscribes to RGB images from an OAK-D camera, performs inference using Ultralytics YOLOv8, estimates object distance using the stereo depth image, visualizes detections, and publishes the results as ROS 2 topics.

The package has been designed with modularity in mind so that new developers can easily:

- Replace the detection model

- Add new classes

- Change visualization

- Integrate with navigation

- Integrate with Behaviour Trees

- Integrate with planning modules

## Features

✓  Real-time YOLOv8 inference

✓  GPU acceleration (CUDA)

✓  Custom trained model

✓  Stereo depth estimation

✓  Bounding box visualization

✓  Confidence estimation

✓  Distance estimation

✓  ROS2 message publishing

✓  Modular architecture

## Software Stack

| Component | Version |
| --- | --- |
| Ubuntu | 22.04 |
| ROS2 | Humble |
| Python | 3.10 |
| PyTorch | CUDA enabled |
| Ultralytics | YOLOv8 |
| OpenCV | 4.x |
| OAK-D Camera | DepthAI |

# 2.  System Architecture

The diagram below shows how image and depth data move through the package, from camera capture to published detections.

Figure 1 — Object detection system architecture

# 3.  Installation

This chapter describes how to install the object detection packages into a ROS 2 Humble workspace.

## Prerequisites

The following software should already be installed on the system.

| Software | Version |
| --- | --- |
| Ubuntu | 22.04 |
| ROS2 | Humble |
| Python | 3.10 |
| OpenCV | 4.x |
| PyTorch | CUDA enabled |
| Ultralytics | YOLOv8 |
| DepthAI ROS Driver V3 | Installed |
| OAK-D Camera | Connected |

## Package Files

The project consists of two ROS2 packages:

object_detection.zip object_detection_msgs.zip

| Package | Description |
| --- | --- |
| object_detection | Performs YOLO inference, distance estimation, visualization, and publishes detections. |
| object_detection_msgs | Defines the custom ROS2 messages used by the object detection package. |

## Create a ROS2 Workspace

If a ROS2 workspace does not already exist, create one.

mkdir -p ~/ros2_ws/src cd ~/ros2_ws

## Copy and Extract the Packages

Copy both zip files into the src directory, then extract them.

cd ~/ros2_ws/src unzip object_detection.zip unzip object_detection_msgs.zip

After extraction, the workspace should contain:

ros2_ws/ └── src/ ├── object_detection/ └── object_detection_msgs/

## Verify Package Structure

The workspace should contain:

ros2_ws/ └── src/ ├── object_detection/ │ ├── package.xml │ ├── setup.py │ ├── object_detection/ │ ├── launch/ │ └── models/ │ └── object_detection_msgs/ ├── package.xml ├── CMakeLists.txt └── msg/ ├── Detection.msg └── DetectionArray.msg

## Build and Source the Workspace

Return to the workspace root and build the packages.

cd ~/ros2_ws colcon build

Expected output:

Starting >>> object_detection_msgs Starting >>> object_detection Finished <<< object_detection_msgs Finished <<< object_detection Summary: 2 packages finished

After every successful build, source the workspace.

source install/setup.bash

To avoid sourcing it manually every time a new terminal is opened, add it to the shell configuration file.

echo "source ~/ros2_ws/install/setup.bash" >> ~/.bashrc source ~/.bashrc

## Verify Installation

Ensure that ROS2 can locate both packages.

ros2 pkg list ¦ grep object_detection

Expected output:

object_detection object_detection_msgs

Verify that the custom messages have been generated correctly.

ros2 interface show object_detection_msgs/msg/Detection

Expected output:

std_msgs/Header header string class_name uint16 class_id float32 confidence float32 distance int32 xmin int32 ymin int32 xmax int32 ymax

If the message definition is displayed correctly, the installation has been completed successfully.

## Continue...

## Complete Installation Workflow

Figure 2 — End-to-end installation workflow

# 4.  Running the Package

# Building the DepthAI ROS Driver

Clone the repository.

cd ~/ros2_ws/src

git clone https://github.com/luxonis/depthai-ros.git

Return to the workspace and build.

cd ~/ros2_ws

colcon build --symlink-install

Source the workspace.

source install/setup.bash

The object detection system can be executed in two different ways depending on the application.

| Method | Recommended for |
| --- | --- |
| Individual nodes | Development, debugging, testing |
| Launch file | Daily operation, deployment |

## Method 1 — Running Individual Nodes

This method provides greater flexibility and is recommended during development, since each node can be started independently.

### Step 1 — Source the ROS2 workspace

source /opt/ros/humble/setup.bash cd ~/ros2_ws source install/setup.bash

### Step 2 — Start the OAK-D camera driver

ros2 launch depthai_ros_driver_v3 driver.launch.py

The camera driver publishes:

| Topic | Description |
| --- | --- |
| /oak/rgb/image_raw | RGB image |
| /oak/stereo/image_raw | Stereo depth image |
| /oak/rgb/camera_info | Camera calibration |

### Step 3 — Verify camera topics

ros2 topic list

Expected topics:

/oak/rgb/image_raw /oak/stereo/image_raw /oak/rgb/camera_info

### Step 4 — Run object detection

Open another terminal.

source /opt/ros/humble/setup.bash cd ~/ros2_ws source install/setup.bash ros2 run object_detection inference_node

### Step 5 — Verify the detection topic

ros2 topic list

Expected output:

/detections

View detections:

ros2 topic echo /detections

Example output:

header: stamp: sec: 1784181506 nanosec: 949150779 detections: - class_name: no_parking confidence: 0.94 distance: 0.80 xmin: 374 ymin: 209 xmax: 410 ymax: 251

## Method 2 — Running Using a Launch File

For deployment, it is preferable to start all required nodes using a single launch file.

Advantages:

- Single command startup

- No need to open multiple terminals

- Easier deployment

- Cleaner workflow

- Standard ROS2 practice

### Launch Architecture

Figure 3 — Launch file architecture

### Launch the Complete System

ros2 launch object_detection object_detection.launch.py

This single command automatically:

- Starts the OAK-D camera driver.

- Initializes the stereo depth stream.

- Starts the YOLO inference node.

- Displays the annotated detection window.

- Publishes detections on /detections.

# 5.  Package Structure

object_detection/ ├── detector.py ├── inference_node.py ├── visualization.py ├── utils.py ├── models/ │ └── last.pt ├── launch/ │ └── object_detection.launch.py ├── package.xml ├── setup.py ├── setup.cfg └── resource/

# 6.  Package Responsibilities

## detector.py

Responsible for:

- Loading the YOLO model

- GPU selection

- Running inference

- Confidence thresholding

- IOU thresholding

- Returning Python dictionaries

Output format:

[ { "class_id": 16, "class_name": "no_parking", "confidence": 0.93, "bbox": [x1, y1, x2, y2] } ]

## visualization.py

Responsible for drawing bounding boxes, labels, confidence, and distance using OpenCV. Example on-screen overlay:

Figure 4 — Example detection overlay

## Continue...

## utils.py

Contains helper formatting functions.

format_confidence() format_distance()

Output:

93.6 % 0.82 m

## inference_node.py

This is the heart of the package. Responsibilities:

- Subscribe to the RGB image

- Subscribe to the depth image

- Run YOLO

- Estimate distance

- Publish ROS messages

- Display visualization

# 7.  ROS Topics

## Subscribed Topics

### RGB Image

| Field | Value |
| --- | --- |
| Topic | /oak/rgb/image_raw |
| Type | sensor_msgs/Image |
| Encoding | bgr8 |

### Stereo Depth

| Field | Value |
| --- | --- |
| Topic | /oak/stereo/image_raw |
| Type | sensor_msgs/Image |
| Encoding | 16UC1 |
| Depth unit | millimetres |

## Published Topics

### Detection Topic

| Field | Value |
| --- | --- |
| Topic | /detections |
| Type | object_detection_msgs/DetectionArray |

Contains:

- Object class

- Confidence

- Distance

- Bounding box

# 8.  Custom Messages

## Why Custom Messages?

ROS already provides image messages but does not provide a message specifically for object detections with confidence, class names, and distance.

Instead of publishing multiple independent topics (class, confidence, distance, bbox), we bundle all related information into one structured message.

Benefits:

- Cleaner API

- Easier to subscribe

- Extensible

- Compatible with downstream planning and navigation nodes

## Detection.msg

std_msgs/Header header string class_name uint16 class_id float32 confidence float32 distance int32 xmin int32 ymin int32 xmax int32 ymax

### Field Descriptions

| Field | Description |
| --- | --- |
| header | ROS timestamp and frame ID |
| class_name | Human-readable object label (e.g., stop, no_parking) |
| class_id | Numeric class ID from the YOLO model |
| confidence | Detection confidence (0.0 - 1.0) |
| distance | Estimated object distance in metres |
| xmin, ymin, xmax, ymax | Bounding box pixel coordinates |

Example:

header: stamp: sec: 1784181506 nanosec: 949150779 frame_id: oak_rgb_camera_optical_frame class_name: no_parking class_id: 16 confidence: 0.94 distance: 0.80 xmin: 374 ymin: 209 xmax: 410 ymax: 251

## DetectionArray.msg

std_msgs/Header header Detection[] detections

### Purpose

Allows multiple detections to be published in a single ROS message for each image frame.

Example:

header: frame_id: oak_rgb_camera_optical_frame detections: - class_name: stop confidence: 0.95 distance: 1.22 - class_name: speed_limit_30 confidence: 0.91 distance: 2.84 - class_name: no_parking confidence: 0.87 distance: 0.79

## Continue...

# 9.  Detection Pipeline

The diagram below summarizes the full per-frame detection and distance-estimation pipeline.

Figure 5 — Per-frame detection pipeline

# 10.  Distance Estimation

The distance estimation process uses the synchronized stereo depth image published by the OAK-D camera.

### Step 1: Compute Bounding Box Centre

The centre pixel of the detected object is calculated as:

### Step 2: Extract a 7×7 Region of Interest (ROI)

A 7×7 window centred at the bounding-box centre is extracted from the depth image to reduce sensitivity to noisy pixels.

### Step 3: Remove Invalid Pixels

Depth values equal to zero are discarded because they represent missing measurements.

### Step 4: Compute Median Depth

The median of the remaining valid depth values is used:

Using the median instead of the mean provides robustness against outliers.

### Step 5: Convert to Metres

The depth image stores values in millimetres. The final distance is: