Vision-Based Traffic Sign Detection

and Depth-Aware Perception Pipeline

Technical Documentation

ROS 2 Traffic Sign Detection Package

# Table of Contents

1. Introduction	..    3

2. System Architecture	 3

2.1 Node and Topic Overview	 3

3. Detection Model – Custom YOLOv8n	 4

3.1 Why YOLOv8n	 4

3.2 Why a Custom-Trained Model Instead of an Off-the-Shelf Model	 4

3.3 Model Configuration	 4

4. Detector Module (detector.py)	 5

4.1 Initialization	 5

4.2 Inference – detect()	 5

5. Depth-Aware Distance Estimation	 5

5.1 Algorithm	 5

6. ROS 2 Integration (inference_node.py)	 6

6.1 Startup	 6

6.2 image_callback – Main Perception Loop	 6

6.3 depth_callback	 6

6.4 publish_detections	 7

7. Visualization Module (visualization.py)	 7

8. Shared Utilities (utils.py)	 7

9. Detection Data Contract	 7

10. Summary	 8

# 1. Introduction

This document describes the design, architecture, and internal working of the ROS 2 perception stack used for real-time traffic sign detection and distance estimation. The system fuses an RGB camera stream with a stereo depth stream from a Luxonis OAK camera to detect traffic signs, classify them, and report their real-world distance to downstream navigation and decision-making nodes.

The perception stack is composed of five Python modules that together form a clean separation between machine-learning inference, ROS 2 middleware plumbing, geometric/depth reasoning, and visualization:

- detector.py – wraps the YOLOv8 model and exposes a framework-agnostic detect() API.

- inference_node.py – the ROS 2 node: subscribes to image/depth topics, orchestrates detection, distance estimation, publishing, and live visualization.

- depth_estimator.py – reserved module for depth-processing utilities (currently a stub; distance logic is implemented inline in inference_node.py, see Section 5).

- visualization.py – draws bounding boxes and confidence/distance labels on frames for on-screen debugging.

- utils.py – shared, framework-agnostic helper functions (formatting, geometry, drawing primitives).

A guiding design principle throughout the codebase is the separation of concerns: detector.py and visualization.py deliberately contain no ROS code, so they can be unit-tested or reused outside of ROS 2 entirely.

# 2. System Architecture

At runtime, data flows through the pipeline in the following order:

| Stage | Component | Description |
| --- | --- | --- |
| 1 | Image acquisition | OAK camera publishes RGB frames on /oak/rgb/image_raw and stereo depth on /oak/stereo/image_raw. |
| 2 | Pre-processing | Frames are rotated 180° (camera is physically mounted inverted) via cv2.rotate. |
| 3 | Inference | Detector.detect() runs the custom YOLOv8n model on the corrected RGB frame. |
| 4 | Depth fusion | For each detected bounding box, a 7×7 depth patch around the box centroid is sampled and reduced to a median distance. |
| 5 | Publishing | Detections (class, confidence, bbox, distance) are packaged into a DetectionArray ROS message and published on /detections. |
| 6 | Visualization | Annotated frame and colourised depth map are rendered locally with OpenCV for live debugging. |

## 2.1 Node and Topic Overview

| Topic | Type | Direction | Purpose |
| --- | --- | --- | --- |
| /oak/rgb/image_raw | sensor_msgs/Image | Subscribed | Raw RGB camera stream (sensor QoS) |
| /oak/stereo/image_raw | sensor_msgs/Image | Subscribed | Raw stereo depth stream (sensor QoS) |
| /detections | object_detection_msgs/DetectionArray | Published | Per-frame list of detected signs with distance |

# 3. Detection Model – Custom YOLOv8n

The detection stage is built around a custom-trained YOLOv8n model (model file: last.pt), fine-tuned specifically on a dataset of traffic signs captured and labelled inside the lab's own test environment, rather than a stock model trained on a generic public dataset such as COCO.

## 3.1 Why YOLOv8n

YOLOv8n (the “nano” variant of the YOLOv8 family) was selected as the base architecture for several concrete engineering reasons relevant to this project:

- Real-time throughput on embedded/edge hardware: the nano variant has the smallest parameter count in the YOLOv8 family, which keeps inference latency low enough to run alongside ROS 2, OpenCV visualization, and depth processing on the same onboard compute.

- Low memory and power footprint: important for a vehicle/robot platform where GPU memory and battery budget are shared with other perception and control nodes.

- Sufficient accuracy for a constrained class set: because the model only needs to recognize the lab's own set of traffic sign classes (a much smaller and more visually consistent label space than general-purpose datasets), the smaller nano backbone reaches high precision without needing the capacity of larger YOLOv8 variants (s/m/l/x).

- Native Ultralytics tooling: YOLOv8 ships with a mature Python API (the ultralytics package used directly in detector.py), simplifying training, export, and inference integration into the ROS 2 node.

## 3.2 Why a Custom-Trained Model Instead of an Off-the-Shelf Model

Generic pretrained detectors (COCO-pretrained YOLO checkpoints, or general traffic-sign datasets such as GTSRB/LISA) are trained on real-world road imagery and public sign standards that do not match the lab's environment. In this project, the lab uses its own scaled-down signage, mounting heights, lighting conditions, and camera geometry (including the intentionally inverted camera mount handled by the 180° rotation step). A model trained specifically on lab-captured images:

- Recognizes the lab's exact sign set and appearance instead of unrelated real-world sign classes, avoiding both false positives on irrelevant classes and false negatives on lab-specific signs the public dataset never saw.

- Is robust to the lab's actual lighting, backgrounds, and camera perspective, since the training images were captured under the same conditions the vehicle/robot operates in.

- Keeps the confidence and IoU thresholds (0.50 / 0.45 in Detector.__init__) meaningful, because they were tuned against the lab's own validation set rather than inherited from an unrelated domain.

In short, the combination of a lightweight YOLOv8n backbone and lab-specific fine-tuning gives the system an accuracy/latency trade-off suited to this exact deployment, which is difficult to obtain from a larger, general-purpose model that was never exposed to the lab's signage during training.

## 3.3 Model Configuration

| Parameter | Value | Purpose |
| --- | --- | --- |
| Model file | last.pt | Final checkpoint from the lab's custom training run, loaded from the object_detection package share directory |
| confidence | 0.50 | Minimum confidence score for a detection to be kept |
| iou | 0.45 | IoU threshold used for non-maximum suppression |
| image_size | 640 | Inference resolution (image is resized to 640×640) |
| device | auto (cuda:0 / cpu) | Automatically selects GPU if torch.cuda.is_available(), else falls back to CPU |

# 4. Detector Module (detector.py)

The Detector class is a thin, ROS-independent wrapper around the Ultralytics YOLO API. It has two responsibilities: load the model once at start-up, and expose a single detect(frame) method that returns plain Python dictionaries rather than framework-specific tensor objects.

## 4.1 Initialization

On construction, Detector resolves the compute device (CUDA if available, otherwise CPU), loads the YOLO checkpoint via YOLO(model_path), moves it to the selected device, and caches the model's class-name mapping (self.class_names) for later lookups.

## 4.2 Inference – detect()

The detect() method performs the following steps for every incoming frame:

- Calls model.predict() with the configured confidence, IoU, image size, and device.

- Guards against empty results (no detections) and returns an empty list early.

- Iterates over result.boxes, extracting the class id, confidence, and bounding box in xy pixel format for each detected object.

- Packages each detection as a dictionary: {class_id, class_name, confidence, bbox}, where bbox is [x1, y1, x2, y2] as integers.

Returning plain dictionaries (instead of Ultralytics' internal Results/Boxes objects) decouples the rest of the pipeline from the YOLO library, so the ROS node, visualizer, and any future consumers only need to understand a simple, stable data contract.

# 5. Depth-Aware Distance Estimation

depth_estimator.py exists as a dedicated module for depth-processing logic, but at the current stage of the project the distance-estimation algorithm is implemented directly inside InferenceNode.estimate_distance() in inference_node.py. Functionally, it works as follows:

## 5.1 Algorithm

- The bounding-box centroid (cx, cy) is computed as the integer midpoint of the box.

- A small 7×7 pixel neighbourhood centred on the centroid is cropped from the most recent depth frame (self.latest_depth), with bounds clamped to the image edges.

- Zero-valued pixels (which represent invalid/unmeasured depth returns from the stereo sensor) are filtered out.

- The median of the remaining valid depth values is taken as the representative depth for that detection, in millimetres, and converted to metres before being attached to the detection dictionary as distance.

- If no valid depth samples exist in the patch (e.g. sensor dropout or the box lies outside the depth frame), distance is reported as None, and later serialized as -1.0 in the published ROS message.

Using a small neighbourhood and the median (rather than a single pixel or the mean) makes the distance estimate resilient to stereo noise and small misalignment between the RGB and depth frames, without requiring a full depth-to-RGB registration step.

# 6. ROS 2 Integration (inference_node.py)

InferenceNode is the single ROS 2 entry point that wires the vision components into the robot's runtime. Its responsibilities are:

## 6.1 Startup

- Resolves the model path from the object_detection package's installed share directory (get_package_share_directory).

- Constructs the Detector with the lab's tuned confidence/IoU/image-size parameters.

- Constructs a Visualizer instance for on-screen debugging.

- Subscribes to the RGB and depth image topics using qos_profile_sensor_data, the QoS profile recommended for high-rate, best-effort sensor streams.

- Creates a DetectionArray publisher on /detections.

## 6.2 image_callback – Main Perception Loop

For every incoming RGB frame, the node:

- Converts the ROS Image message to an OpenCV BGR array via cv_bridge.

- Rotates the frame 180° to correct for the camera's inverted physical mounting.

- Waits until at least one depth frame has been received before running detection, avoiding distance estimation against stale/empty depth data.

- Runs Detector.detect() to obtain the current frame's detections.

- Attaches a distance estimate to each detection via estimate_distance().

- Publishes the enriched detections as a DetectionArray via publish_detections().

- Logs a human-readable summary line per detection (class, confidence, distance) for live debugging.

- Draws annotated bounding boxes/labels with Visualizer.draw() and displays the frame in an OpenCV window.

## 6.3 depth_callback

Every incoming depth frame is converted with passthrough encoding (preserving raw depth values), rotated 180° to match the RGB frame's orientation, cached in self.latest_depth for use by estimate_distance(), and separately normalized and colour-mapped (COLORMAP_JET) purely for a live depth-visualization window.

## 6.4 publish_detections

Converts the internal list of detection dictionaries into a DetectionArray ROS message, copying the incoming image header (for timestamp/frame_id consistency), and mapping distance = None to a sentinel value of -1.0 so that downstream consumers can distinguish “no valid depth” from a genuine zero-distance reading.

# 7. Visualization Module (visualization.py)

The Visualizer class renders detections onto a copy of the input frame for debugging and demonstration purposes, without mutating the original frame used for downstream processing.

- Bounding box colour is confidence-coded via _get_color(): green for ≥80%, yellow for 60–80%, red for below 60%, giving an at-a-glance sense of detection reliability.

- A multi-line label (class name, confidence percentage, and distance when available) is drawn above each bounding box on a solid background matching the box colour, sized dynamically to fit the longest line.

- Text formatting is delegated to utils.format_confidence() and utils.format_distance() to keep string formatting consistent across the codebase.

# 8. Shared Utilities (utils.py)

utils.py collects small, dependency-light helper functions used across the package:

| Function | Purpose |
| --- | --- |
| rotate_image_180(frame) | Rotates a frame 180°; used to correct for the inverted camera mount. |
| bbox_center(bbox) | Returns the integer centroid (cx, cy) of a [xmin, ymin, xmax, ymax] box. |
| format_confidence(confidence) | Formats a 0–1 confidence value as a percentage string. |
| format_distance(distance) | Formats a distance in metres to two decimal places. |
| draw_label(image, text, x, y, color) | Draws a single text label with a black background box for readability. |
| clamp(value, minimum, maximum) | Clamps a numeric value to a [minimum, maximum] range. |

# 9. Detection Data Contract

Every detection produced by Detector.detect() and consumed downstream follows a single, stable dictionary shape:

{

"class_id":    int,       # index into the model's class list

"class_name":  str,       # human-readable label, e.g. "stop_sign"

"confidence":  float,     # 0.0 - 1.0

"bbox":        [x1, y1, x2, y2],  # pixel coordinates, ints

"distance":    float | None,      # metres; added by InferenceNode

}

Keeping this contract independent of both the Ultralytics API and any ROS message type is what allows detector.py and visualization.py to remain reusable outside of ROS 2.

# 10. Summary

This pipeline combines a lightweight, lab-customised YOLOv8n detector with simple but effective stereo-depth fusion to deliver real-time, distance-aware traffic sign detection inside a ROS 2 node. The architecture keeps machine-learning inference (detector.py), visualization (visualization.py), and generic helpers (utils.py) free of ROS-specific code, isolating all middleware concerns inside inference_node.py. This separation, combined with a model trained specifically on the lab's own signage and operating conditions, gives the system both the accuracy and the real-time performance the deployment requires.