Technical Document

LiDAR-Based Obstacle Detection Node (ROS 2)

| Field | Detail |
| --- | --- |
| Module | obstacle_detector.py |
| Node name | obstacle_detector |
| Framework | ROS 2 (rclpy) |
| Primary input | sensor_msgs/LaserScan on /scan |
| Primary output | obstacle_msgs/Obstacle on /front_obstacle |
| Reference frames | lidar_link (sensor) → odom (world) |

# 1. Overview

This document describes the algorithm and implementation of the ObstacleDetector ROS 2 node, which consumes 2D LiDAR scans and detects the closest obstacle within a configurable forward field of view (FOV). The emphasis of this document is the detection algorithm itself — the angular filtering, clustering, nearest-obstacle selection, and geometric feature extraction that together turn a raw range array into a usable obstacle estimate.

The node clusters raw range readings, isolates the nearest cluster, computes its geometric properties (center, width, distance, bearing), transforms these properties into the odometry frame, and publishes both a structured obstacle message and RViz visualization markers.

## 1.1 Functional Summary

- Subscribes to raw 2D LiDAR scans and restricts processing to a forward angular window.

- Groups adjacent scan points into clusters using a Euclidean distance threshold.

- Selects the cluster nearest to the sensor as the obstacle of interest.

- Derives obstacle center, edge points, width, distance, and bearing angle.

- Transforms obstacle geometry from the sensor frame (lidar_link) to the world frame (odom) via tf2.

- Publishes the obstacle as a structured message, an RViz cube marker, and a debug FOV marker.

- Renders a live 2D Matplotlib plot of scan points, obstacle edges, and obstacle center.

# 2. Background: Prior Approaches Evaluated

Before settling on 2D LiDAR-based clustering, two alternative obstacle-detection approaches were evaluated against the project's onboard compute budget. Both were capable of more generic obstacle detection but were ultimately rejected on compute-cost grounds.

## 2.1 3D Camera-Based Obstacle Detection

A depth-camera pipeline was evaluated first. This approach required ground plane elimination (segmenting and removing the ground/floor plane from the 3D point cloud before any obstacle could be isolated), in addition to the standard point cloud filtering and downstream clustering. The added ground plane estimation step, combined with the volume of 3D point data to process per frame, made this pipeline too computationally heavy for the available onboard compute.

## 2.2 YOLOv8-Based Object Detection

A second approach used a YOLOv8 object detection model, with the intent of pairing 2D detections with depth estimation to recover obstacle distance — which would have given a generic, class-agnostic obstacle detector rather than one limited to geometric clustering. This was rejected for two reasons:

- The model was not trained on the lab setup / deployment environment, and did not generalize to it — detections were unreliable on the actual test floor and objects used.

- Combined with depth estimation for distance computation, the model was too heavy for the available compute, in the same way the 3D camera pipeline was.

## 2.3 Resulting Design Decision

The goal was a generic obstacle detector with distance estimation, but both routes to that goal (3D point-cloud segmentation and vision-based detection + depth) exceeded the available compute budget. This motivated the shift to a lightweight 2D LiDAR-based obstacle detection approach: a purely geometric clustering algorithm over a 2D range array, with no learned model and no 3D point cloud processing. This trades away object-class awareness (the LiDAR approach detects “something is there”, not “what it is”) for a detector that is cheap enough to run continuously on the onboard compute and does not depend on training-data coverage of the deployment environment.

| Approach | Distance Estimation | Generic (class-agnostic) | Compute Cost | Outcome |
| --- | --- | --- | --- | --- |
| 3D camera + ground plane removal | Native (depth) | Yes | High (ground plane segmentation + 3D processing) | Rejected — too heavy |
| YOLOv8 + depth estimation | Via depth model | Yes (by design) | High (model inference + depth) | Rejected — heavy, and untrained on lab setup |
| 2D LiDAR clustering (adopted) | Native (range) | No (geometry only) | Low | Adopted |

# 3. Node Architecture (Summary)

The following is a brief summary of the surrounding ROS node structure; the algorithm itself (Section 4) is the focus of this document.

The node is implemented as a single rclpy.node.Node subclass. Initialization sets up publishers, a subscriber, detection parameters, and a Matplotlib figure. All detection logic executes inside a single scan callback, invoked once per incoming LaserScan message. Topics: subscribes to /scan (sensor_msgs/LaserScan, BEST_EFFORT/KEEP_LAST(10)); publishes /front_obstacle (obstacle_msgs/Obstacle), /obstacle_marker (FOV cone, LINE_LIST) and /detected_obstacle_marker (obstacle cube). A tf2_ros.Buffer/TransformListener pair maintains the odom ← lidar_link transform used to express results in the world frame.

## 3.1 Detection Parameters

Three parameters govern the algorithm's behavior (see Section 4 for how each is used). They are currently hard-coded in __init__ rather than declared via declare_parameter, so they are not runtime-configurable without a code change.

| Attribute | Value | Meaning |
| --- | --- | --- |
| self.front_fov | radians(20) ≈ 0.349 rad | Half-angle of the forward detection cone (±20° total 40°) |
| self.fov_offset | 0.0 rad | Angular bias applied before the FOV check (e.g. to re-center a mounted sensor) |
| self.cluster_threshold | 0.20 m | Max Euclidean gap between consecutive points to remain in the same cluster |

# 4. Core Detection Algorithm (scan_callback)

This is the central contribution of the node and the focus of this document: a lightweight, purely geometric pipeline that turns one LaserScan message into a single nearest-obstacle estimate with no learned model and no 3D point cloud. It runs entirely inside scan_callback, in five conceptual stages, executed once per incoming scan.

raw ranges[] ¦ Stage A v cartesian points within +/-front_fov, range-valid -- O(n) ¦ Stage B v sequential distance-threshold clustering -- O(n) ¦ Stage C v nearest cluster to sensor origin (by midpoint range) -- O(k), k = #clusters ¦ Stage D v center / width / distance from cluster edge points -- O(1) ¦ Stage E v lidar_link -> odom transform (tf2) -- O(1) x3

Overall cost is O(n) in the number of raw range samples, with no iterative optimization, no model inference, and no per-point neighbor search beyond the immediately preceding point — this is what makes the approach cheap enough for the available onboard compute, at the cost of being geometry-only (no object classification).

## 4.1 Stage A — Polar-to-Cartesian Conversion with FOV Gating

Each range reading is converted to a bearing angle using the scan's angle_min and angle_increment, then offset by fov_offset. Readings outside ±front_fov are discarded. Readings equal to or outside [range_min, range_max] are discarded as invalid.

for i, r in enumerate(scan.ranges): angle = scan.angle_min + i * scan.angle_increment angle += self.fov_offset if abs(angle) > self.front_fov: continue if scan.range_min < r < scan.range_max: x = r * math.cos(angle) y = r * math.sin(angle) points.append((x, y))

A minimum of 5 valid points is required to proceed; otherwise the callback returns early. This guards against sparse/noisy scans producing spurious single-point detections.

Rationale: restricting to a narrow forward cone (±20° by default) before any further processing is what keeps this stage cheap — the algorithm only ever does downstream work on the small subset of the ~360° scan that is actually relevant to forward obstacle avoidance, rather than clustering the full scan and filtering afterwards.

## 4.2 Stage B — Sequential Distance-Threshold Clustering

Points are already ordered by increasing angle (scan order). The algorithm performs a single left-to-right pass, grouping consecutive points into a cluster while the Euclidean distance between successive points stays below cluster_threshold. A gap larger than the threshold closes the current cluster and starts a new one. Clusters with 5 or fewer points are discarded as noise.

clusters = [] current = [] for p in points: if not current: current.append(p); continue last = current[-1] d = math.hypot(p[0]-last[0], p[1]-last[1]) if d < self.cluster_threshold: current.append(p) else: if len(current) > 5: clusters.append(current) current = [p] if len(current) > 5: clusters.append(current)

This is a 1D sequential clustering over angularly-ordered points, not a full spatial clustering algorithm (e.g. DBSCAN, which considers arbitrary neighbor relationships and typically costs O(n log n) or worse with a spatial index). Because it only ever compares a point to the one immediately before it in scan order, it is a single O(n) pass with O(1) work per point — no distance matrix, no neighbor search, no iterative refinement. This is precisely the property that makes it cheap enough to be the replacement for the point-cloud clustering used in the rejected 3D-camera approach.

Trade-off: because it is a 1D sequential pass rather than full 2D spatial clustering, it cannot separate two objects that happen to be angularly adjacent with no range gap between them, and it can split a single curved or angled surface into multiple clusters if the point-to-point spacing along that surface exceeds cluster_threshold at any point (e.g. a flat surface viewed at a shallow angle, where consecutive ray hits are spaced farther apart).

## 4.3 Stage C — Nearest-Obstacle Selection

Among all retained clusters, the algorithm selects the one whose midpoint (by index into the cluster's point list) is closest to the sensor origin. This is an O(k) scan over the (small) list of clusters, each requiring one O(1) distance evaluation:

obstacle = min( clusters, key=lambda c: math.hypot(c[len(c)//2][0], c[len(c)//2][1]) )

The first and last points of the selected cluster are treated as its left and right edges for the next stage. Using the midpoint of the cluster as the ranking key (rather than, say, the nearest point in the cluster) makes the ranking robust to a single noisy near/far outlier point at either edge of a cluster.

## 4.4 Stage D — Geometric Feature Extraction

From the two edge points of the selected cluster, the algorithm computes the obstacle's center (midpoint of the two edges), width (edge-to-edge Euclidean distance), and distance (sensor-to-center Euclidean distance). This is the step that gives the algorithm its distance-estimation capability — the same capability that, in the rejected approaches, required either a depth camera or a separate depth-estimation model:

left, right = obstacle[0], obstacle[-1] cx = (left[0] + right[0]) / 2 cy = (left[1] + right[1]) / 2 width = math.hypot(right[0]-left[0], right[1]-left[1]) distance = math.hypot(cx, cy)

Because width and center are derived only from the two endpoints of the cluster, this approximates the obstacle as a straight chord between its outermost detected points. Concave or highly curved obstacles will have their true width and center under- or over-estimated by this method. This is the main geometric approximation the algorithm makes in exchange for O(1) feature extraction.

## 4.5 Stage E — Frame Transformation (lidar_link → odom)

Center, left, and right points are each transformed from the sensor frame into odom via the transform_point helper (Section 5), so that the algorithm's output is usable directly in the vehicle's world/navigation frame rather than only relative to the sensor. The obstacle message is populated exclusively with odom-frame coordinates; distance and angle are recomputed in odom rather than reusing the sensor-frame values, so they reflect the vehicle's odometry-frame position and heading:

center_odom = self.transform_point(cx, cy) left_odom = self.transform_point(left[0], left[1]) right_odom = self.transform_point(right[0], right[1]) if center_odom is None: return # left/right may still be None downstream if TF fails intermittently

Implementation note: the callback returns only when center_odom is None. left_odom and right_odom are not independently null-checked before being read into msg.left_x/left_y/right_x/right_y, so a transient TF failure limited to just the left or right lookup (while the center lookup succeeds) would raise an exception when indexing a None result.

## 4.6 Stage F — Publishing the Result

On a successful center transform, the node publishes, in order: the RViz obstacle cube marker, the Obstacle message on /front_obstacle. The FOV boundary marker on /obstacle_marker is published on every callback regardless of transform success, since it only depends on static parameters.

| Field | Source | Frame |
| --- | --- | --- |
| center_x, center_y | center_odom | odom |
| left_x, left_y | left_odom | odom |
| right_x, right_y | right_odom | odom |
| distance | hypot(center_odom.x, center_odom.y) | odom |
| width | sensor-frame chord length | frame-invariant (Euclidean distance) |
| angle | atan2(center_odom.y, center_odom.x) | odom |

Note: msg.center_x / msg.center_y are assigned twice in the source (once immediately after msg construction, once again under the “all values in odom frame” block) with identical values; the second assignment is redundant but harmless.

# 5. Coordinate Transformation Helper

transform_point(x, y) wraps a single-point tf2 lookup and transform, returning a tuple or None on failure:

def transform_point(self, x, y): p = tf2_geometry_msgs.PointStamped() p.header.frame_id = "lidar_link" p.header.stamp = self.get_clock().now().to_msg() p.point.x, p.point.y, p.point.z = x, y, 0.0 try: tf = self.tf_buffer.lookup_transform( "odom", "lidar_link", rclpy.time.Time()) out = tf2_geometry_msgs.do_transform_point(p, tf) return (out.point.x, out.point.y) except Exception: return None

- rclpy.time.Time() (zero/empty time) requests the latest transform in the buffer rather than the transform at the scan's own timestamp — simpler, but not point-in-time accurate if the vehicle moves quickly relative to TF publish rate.

- The stamp set on the PointStamped header (current wall time) is not actually used by lookup_transform for the empty-time query, so it has no effect on which transform is applied here.

- All exceptions are caught broadly; TF-specific exceptions (LookupException, ExtrapolationException, ConnectivityException) are not distinguished, which makes failure-cause diagnosis harder from logs alone.

# 6. Visualization Marker Implementation

## 6.1 FOV Boundary Marker (publish_marker)

Publishes a LINE_LIST marker in the lidar_link frame with two rays from the origin at +front_fov and –front_fov, each of fixed length 5.0 m, colored green. This gives a constant visual reference for the detection cone in RViz regardless of whether an obstacle is currently detected.

m.type = Marker.LINE_LIST m.scale.x = 0.02 m.color.g = 1.0; m.color.a = 1.0 length = 5.0 left.x = length*cos(front_fov); left.y = length*sin(front_fov) right.x = length*cos(-front_fov); right.y = length*sin(-front_fov) m.points = [origin, left, origin, right]

## 6.2 Obstacle Marker (publish_obstacle_marker)

Publishes a CUBE marker in the odom frame, positioned at the transformed obstacle center (z fixed at 0.5 m), sized using the computed width along the marker's Y-scale (X and Z scale are fixed at 0.5 m and 1.0 m respectively), colored red with 0.8 alpha. This marker is only published when the center transform succeeds.

# 7. Local Debug Visualization (Matplotlib, Supplementary)

A non-ROS, process-local Matplotlib figure is created in the constructor with interactive mode enabled (plt.ion()), fixed axis limits (X: –1 to 5 m, Y: –3 to 3 m), and equal aspect ratio. Three plot artists are updated per callback:

- scan_plot — scatter of all FOV-filtered scan points.

- obstacle_edge — line between the selected cluster's left and right edge points.

- obstacle_center — marker at the computed obstacle center.

An on-plot text annotation reports live distance and width. The figure is redrawn with fig.canvas.draw() / flush_events() every callback, and all ray-line artists from the previous callback are explicitly removed and rebuilt to avoid unbounded artist accumulation.

This view is useful for bench-testing but ties the node to a display; running headless (e.g. on a robot with no X server) will require removing or guarding this block.

# 8. Data Flow Summary

/scan (LaserScan) ¦ v [FOV filter + polar->cartesian] --front_fov, fov_offset ¦ v [sequential clustering] --cluster_threshold ¦ v [nearest-cluster selection] ¦ v [center / width / distance] (lidar_link frame) ¦ v [transform_point x3] lidar_link -> odom (tf2_ros) ¦ +--> /obstacle_marker (FOV cone, every callback) +--> /detected_obstacle_marker (obstacle cube, on TF success) +--> /front_obstacle (Obstacle msg, on TF success)

# 9. Implementation Notes & Observations

The following are implementation-level observations relevant to maintenance or extension of this node:

- Parameters (front_fov, fov_offset, cluster_threshold) are hard-coded rather than declared with declare_parameter, so they cannot be tuned via ROS parameter files or launch arguments without a code change.

- Clustering is a single angular pass with a fixed distance threshold; it does not perform full 2D spatial clustering, so results depend on scan point ordering and density.

- Obstacle width/center use only the two cluster endpoints, approximating the obstacle as a chord rather than fitting the full point set.

- The Matplotlib debug view runs in-process on every callback and requires a display; it should be feature-flagged or removed for headless/production deployment.

- msg.center_x and msg.center_y are set twice with identical values — a minor redundancy with no functional impact.

- The algorithm is intentionally geometry-only and class-agnostic — it reports “nearest obstacle geometry”, not object identity. This is a direct consequence of the compute-budget trade-off in Section 2 (Section 2.3) rather than an oversight; adding classification would mean reintroducing a model (YOLOv8-class or similar) and its associated compute cost.