APRILTAG-BASED

LOCALIZATION

SETUP AND VERIFICATION GUIDE

Build, launch, and pose-verification procedure for the ips localization package

ROS 2  •  Overhead Camera  •  AprilTag

Version 1.0

Date: July 2026

Table of Contents

1.  Package Information

2.  System Architecture

3.  Workspace Setup

4.  Build and Run Workflow

5.  Verifying Detection

6.  Viewing the Detected Pose

# 1.  Package Information

Package name: ips

If the package is provided as a ZIP file:

1.  Extract (unzip) the package.

2.  Copy the extracted ips package into the src directory of your ROS 2 workspace.

# 2.  System Architecture

The diagram below shows how the overhead camera feed is processed into per-vehicle pose topics.

Figure 1 — AprilTag localization system architecture

# 3.  Workspace Setup

If you do not already have a ROS 2 workspace, create one using the following commands:

mkdir -p ~/ros2_ws/src cd ~/ros2_ws colcon build source install/setup.bash

Note: it is recommended to add the following line to your ~/.bashrc so the workspace is sourced automatically whenever a new terminal is opened:

source ~/ros2_ws/install/setup.bash

# 4.  Build and Run Workflow

The diagram below summarizes the full sequence from copying the package to verifying the published pose.

Figure 2 — Setup, build, and verification workflow

## Building the Package

After copying the ips package into ~/ros2_ws/src, rebuild the workspace:

cd ~/ros2_ws colcon build source install/setup.bash

## Running the Localization Node

Launch the AprilTag localization node:

ros2 run ips ips_node

# 5.  Verifying Detection

After starting the node, an output window similar to Figure 3 will appear.

Place the vehicle with the corresponding AprilTag ID within the camera's field of view. Once the tag is detected, the localization system will estimate and display the vehicle's pose.

Figure 3 — AprilTag-based localization, live detection view

# 6.  Viewing the Detected Pose

To monitor the estimated pose of a detected tag, use the following command:

Replace the tag number with the actual vehicle's AprilTag ID.

ros2 topic echo /apriltag/tag1/pose

For example:

| Vehicle ID | Topic |
| --- | --- |
| 1 | /apriltag/tag1/pose |
| 2 | /apriltag/tag2/pose |
| 3 | /apriltag/tag3/pose |

If the tag is successfully detected, the terminal will continuously publish the pose information, including the tag's position and orientation.

Example output:

Figure 4 — Estimated pose of a detected tag

This confirms that the AprilTag has been detected successfully and that the localization node is publishing the vehicle's pose.