Sensor Fusion & EKF Localization Stack

Implementation Runbook

Bring the stack up in the order below. Each stage includes a verification block — run those checks and confirm the expected result before starting the next stage. Use a separate terminal (or tmux/tab) per command.

CAUTION: The current ekf_local.yaml only feeds Visual Odometry (/visual_odometry) into the local EKF. The VESC wheel-odometry block and the IMU block are present in the YAML but commented out. Steps 1 and 6 below still start those nodes (so their topics exist and can be inspected), but the EKF will not currently fuse them.

Step 0  Workspace Setup — program_bringup Package

The EKF launch file and YAML configs (ekf.launch.py, ekf_local.yaml, ekf_global.yaml) are packaged as program_bringup.zip. Unzip it into your ROS 2 workspace's source folder and build it before any of the steps below.

Commands:

mkdir -p ~/ros2_ws/src

cd ~/ros2_ws/src

unzip /path/to/program_bringup.zip -d .

Copy the unzipped package in your workspace’s /src

cd ~/ros2_ws

colcon build --packages-select program_bringup

source install/setup.bash

Run the source command in every new terminal you open for the steps below (or add it to your ~/.bashrc).

NOTE: robot_localization ships as a standard ROS 2 package — it does not need to be separately installed. As long as your ROS 2 distro is sourced, ros2 launch program_bringup ekf.launch.py can find the ekf_node executable it depends on.

Verify before continuing:

ros2 pkg list | grep program_bringup

Expect: program_bringup listed, confirming the build succeeded and the package is on the ROS 2 path.

ros2 pkg prefix program_bringup

Expect: a path under your workspace's install/ directory — if this errors, source install/setup.bash first.

Step 1  VESC Driver

Command:

ros2 launch vesc_driver vesc_driver_node.launch.py

Starts the low-level driver that talks to the VESC motor controller over serial/CAN and publishes raw motor/sensor state (RPM, current, voltage).

Verify before continuing:

ros2 node list | grep vesc

Expect: a vesc_driver_node (or similar) listed.

ros2 topic list | grep vesc

Expect: a /sensors/core or /vesc/... state topic.

ros2 topic hz /sensors/core

Expect: a steady non-zero publish rate. If it hangs with no output, the serial/CAN connection to the VESC is not established — check wiring/permissions before continuing.

Step 2  VESC Kinematic Odometry

Command:

ros2 run vesc_kinematic_odom vesc_kinematic_odom

Converts raw VESC motor state from Step 1 into wheel odometry (/odom/vesc) using the vehicle's kinematic model.

Verify before continuing:

ros2 topic hz /odom/vesc

Expect: non-zero rate. If zero, this node isn't receiving Step 1's topic — check the topic name it subscribes to matches Step 1's output.

ros2 topic echo /odom/vesc --once

Expect: a nav_msgs/Odometry message with plausible x/y/yaw and covariance fields populated (not all zeros).

NOTE: As of the current ekf_local.yaml, /odom/vesc is not consumed by the EKF (its block is commented out). This step still confirms the sensor chain is healthy independently of the filter.

Step 3  DepthAI (OAK-D) Camera Driver

Command:

ros2 launch depthai_ros_driver_v3 driver.launch.py

Brings up the OAK-D camera driver: image streams, calibration, and (depending on config) stereo depth.

Verify before continuing:

ros2 node list | grep -i depthai

Expect: the depthai driver node listed.

ros2 topic list | grep -i image

Expect: RGB/stereo image topics present.

ros2 topic hz /oak/rgb/image_raw   # adjust topic name to your config

Expect: non-zero rate matching the configured camera FPS.

Step 4  OAK-D Visual Odometry Node

Command:

ros2 run oakd_vo oakd_vo_node

Computes visual odometry from the camera stream in Step 3 and publishes it — this is the source of /visual_odometry consumed by the local EKF.

Verify before continuing:

ros2 topic hz /visual_odometry

Expect: non-zero rate. If zero, confirm the image topic name this node subscribes to matches Step 3's actual output topic.

ros2 topic echo /visual_odometry --once

Expect: a nav_msgs/Odometry message with a moving x/y as the robot is nudged, and yaw changing when it's rotated.

Step 5  AprilTag / IPS Odometry Node

Command:

ros2 run apriltag_odom apriltag_odom

Detects AprilTags (or other IPS markers) and publishes an absolute position estimate — this is the source of /odometry/ips consumed by the global EKF's odom1 input.

Verify before continuing:

ros2 topic hz /odometry/ips

Expect: intermittent, not necessarily continuous — this topic only publishes when a tag is actually in view. Zero output while a tag is visible means detection isn't working; check camera framing and tag IDs.

ros2 topic echo /odometry/ips --once

Expect: x/y in map-frame coordinates that correspond to the tag's known position.

Step 6  IMU Node

Command:

ros2 run bno_test_pkg imu_node

Publishes raw IMU orientation/angular-velocity data from the BNO sensor.

Verify before continuing:

ros2 topic hz /imu

Expect: a high, steady rate (IMUs typically publish at 50-200+ Hz).

ros2 topic echo /imu --once

Expect: a sensor_msgs/Imu message with a valid orientation quaternion (not all zeros) and non-crazy angular_velocity values at rest.

NOTE: As with Step 2, the IMU block in ekf_local.yaml is currently commented out, so /imu is not yet fused. This step only confirms the sensor itself is healthy.

Step 7  EKF Local + Global Filters

Command:

ros2 launch program_bringup ekf.launch.py

Starts both EKF nodes defined in ekf.launch.py: ekf_local_node (root namespace, publishes odom→base_link on /odometry/filtered) and ekf_global_node (namespace /global, publishes map→odom on /global/odometry/filtered). This step depends on Steps 3-5 already running, since odom0 for the local filter and odom1 for the global filter come from those nodes.

Verify before continuing:

ros2 node list | grep ekf

Expect: /ekf_local_node and /global/ekf_global_node both listed.

ros2 topic hz /odometry/filtered

Expect: non-zero rate at ~10 Hz (the configured frequency).

ros2 topic hz /global/odometry/filtered

Expect: non-zero rate at ~10 Hz.

ros2 topic echo /diagnostics --once

Expect (print_diagnostics: true is set): diagnostic status entries for each EKF node, each reporting OK rather than a stale-sensor or timeout warning.

ros2 run tf2_ros tf2_echo map base_link

Expect: a valid, non-static transform once both filters are running — confirms the full map→odom→base_link chain is connected.

CAUTION: If tf2_echo map base_link fails or times out, check that both nodes actually have publish_tf: true active and that ekf_global.yaml's top-level key (/global/ekf_global_node) matches the namespace: "global" set in ekf.launch.py — a mismatch here silently drops the global filter's parameters.

Step 8  Planner / Visualizer

Command:

ros2 run planner_visualizer visualizer

Starts the visualization tool used to inspect the planner and the fused localization output. Run this last, once Step 7's verification has passed, so there is a valid map→base_link transform for it to display against.

Verify before continuing:

ros2 node list | grep visualizer

Expect: the visualizer node listed and no repeated TF-lookup errors in its log output.

# Summary Checklist

| Step | Command | Key topic to confirm |
| --- | --- | --- |
| 0 | colcon build (program_bringup) | ros2 pkg list ¦ grep program_bringup |
| 1 | vesc_driver_node.launch.py | /sensors/core |
| 2 | vesc_kinematic_odom | /odom/vesc |
| 3 | depthai driver.launch.py | camera image topics |
| 4 | oakd_vo_node | /visual_odometry |
| 5 | apriltag_odom | /odometry/ips |
| 6 | bno_test_pkg imu_node | /imu |
| 7 | ekf.launch.py | /odometry/filtered, /global/odometry/filtered |
| 8 | planner_visualizer | visual confirmation of localization |