VISUAL ODOMETRY

EVALUATION GUIDE

Build, launch, and verification procedure for the OAK-D visual odometry package

ROS 2  •  OAK-D Camera

Version 1.0

Date: July 2026

Table of Contents

1.  Overview

2.  System Architecture

3.  Prerequisites

4.  Installation

5.  Build, Launch, and Verify Workflow

6.  Expected Output

7.  Troubleshooting

# 1.  Overview

This document describes the steps required to build and run the OAK-D Visual Odometry package and verify that the visual odometry data is being published successfully.

# 2.  System Architecture

The diagram below shows how RGB and depth data flow from the camera through the visual odometry node to downstream consumers.

Figure 1 — Visual odometry system architecture

# 3.  Prerequisites

✓  ROS 2 workspace is already created.

✓  depthai_ros_driver_v3 package is installed and working.

✓  OAK-D camera is connected to the system.

✓  ROS 2 environment is properly configured.

# 4.  Installation

Copy the oakd_vo package into the src directory of your ROS 2 workspace.

<ros2_workspace>/ └── src/ ├── oakd_vo/ └── ...

# 5.  Build, Launch, and Verify Workflow

The diagram below summarizes the full sequence from building the package to observing live odometry updates.

Figure 2 — Build, launch, and verification workflow

## Build the Package

Navigate to the root of your ROS 2 workspace and build the complete workspace:

colcon build --symlink-install

...or build only the Visual Odometry package:

colcon build --packages-select oakd_vo

## Source the Workspace

After the build completes successfully, source the workspace.

source install/setup.bash

## Launch the OAK-D Camera Driver

Start the OAK-D camera by launching the DepthAI ROS driver, and wait until it starts publishing the required RGB and depth topics.

ros2 launch depthai_ros_driver_v3 driver.launch.py

## Run the Visual Odometry Node

Open a new terminal, source the workspace again, and execute:

source install/setup.bash ros2 run oakd_vo oakd_vo_node

## Verify Visual Odometry Output

Once the node is running successfully, it will begin publishing the estimated robot pose on /visual_odometry. Verify the topic using:

ros2 topic list ros2 topic echo /visual_odometry

You should observe a continuous stream of odometry messages containing the estimated position and orientation of the camera.

# 6.  Expected Output

If the system is operating correctly:

✓  The OAK-D camera launches without errors.

✓  The Visual Odometry node starts successfully.

✓  The /visual_odometry topic is created.

✓  Odometry messages are continuously published as the camera moves.

✓  The estimated trajectory changes according to the camera motion.

Example:

Figure 3 — Example /visual_odometry message

# 7.  Troubleshooting

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| /visual_odometry topic not available | OAK-D driver not running | Launch the camera driver before starting the VO node. |
| No odometry updates | Camera is stationary | Move the camera to generate visual features for motion estimation. |
| Build errors | Workspace not sourced or dependencies missing | Rebuild the workspace and source install/setup.bash. |
| Camera topics | OAK-D not detected | Verify USB connection and ensure the camera driver launches successfully. |

ros2 launch vesc_driver vesc_driver_node.launch.py